# Lab 1

Orientation microbiology lab (Tools and Equipment)

Prepared by

# M.SC Hawraa D.salman

Lab-1-

# Medical Microbiology classified into:

- Bacteriology is Study of bacteria

- Mycology is study of fungi

- Virology is study of viruses

- Parasitology is study of protozoan & helminthes

5- Immunology is study of resistance to disease

# Tools and Equipment

Tools

1-Loop

The loop is used in the cultivation of microbes on plates by

Transferring inoculums for streaking. Touching a broth or

a culture plate will gather enough microbes (0.01ml) for

Inoculation. The inoculation loop is sterilized with flame or another heat source.

2-Can

Used for preserve the pipette from any contamination, sterilized with pipettes by autoclave

3-Pipette

Used for transfer of cultured and uncultured broth from tube or flask to other and placed in can sterilized by autoclave inside the can.

4-Spreader (L-shape)

Used for spreading bacterial cell on surface of solid medium in petriplate ,before using placed in alcohol and then sterilized by flame of burner.

5-Petri-Dish(Petri-plate)

Used for place the solid medium in it, glass  petri-dish used for many times and sterilized by oven or autoclave ,while sterilized plastic plates used for one time.

6-Swab

Used for swabbing bacterial cells on the surface of solid medium in Petri plate, must be placed in test tube and sterilized by autoclave, it used for one time.

7-Test tube

Used to place the liquid or solid or semisolid medium for stabbing or placed as slant for culture of bacteria ,it sterilized by autoclave.

8-Needle

Used for transfer of bacterial cells to a solid medium or semi-solid medium by stabbing, sterilized by the flame of burner before and after use.

9-Slide

Used for examination of bacterial smear under microscope, it used for one time.

10-Cover-Slips

Placed on the slide, the bacterial smear may be between the cover and the slide, it used for one time.

11-Flask

Used for place cultured and uncultured broth in it ,

sterilized after plugs with cotton by autoclave.

12-Cotton plugs

A piece of coiled cotton used to close the upper part of flasks and tubes.

13-Beaker

Used for graduated the volume of liquids. Sterilized by oven.

14-Cylinder(Graduated Cylinder)

Used for graduated the volume of liquids, sterilized by oven.

15-Washing bottle

Used to fill with liquid (specially distilled water)for washing

and homogenizing the glass wares and washing the slide

during staining , don’t need sterilization.

16-pH paper

Used to know the pH of the medium or any liquids

17-Rack

May be wooden, metallic or plastic used to stand and hold the tube.

18-Burner

May be gaseous or alcoholic , used for sterilized the loop , needle

and other metallic tools by flame (dry heat sterilization).

Equipments

1-Autoclave

Wet heat sterilization= death by protein denaturation

It’s an equipment with:

High temperature (121C˚)

High pressure 1 atm(15 pound/inch2) used to sterilize media

(With sugar for 10min)

(Uncultured media for 15min),

(Cultured media and contaminated glass wares for 30 min)

2-Oven

Dry heat sterilization=death by oxidation

Equipment with high temperature only (180C˚) for (90 min)

used to sterilized some of metallic tools and glass wares

3-Incubator

Is a device used to grow and maintain microbiological cultures or cell

cultures. The incubator maintains optimal temperature ,humidity and

other conditions such as the carbon dioxide (CO2) and oxygen content

of the atmosphere inside

4-Refrigerator

Used to maintain the sterilized media and broth when not

used to avoid the contamination , and also to preserve the

bacterial culture for long time by preventing the growth at 4C˚.

5-Biosafety cabinet (BSC)

Also called a biological safety cabinet or microbiological safety cabinet is aenclosed,ventilated laboratory workspace for safely working with materials contaminated with (or potentially contaminated with) pathogens

6-Waterbath

Is laboratory equipment made from a container filled with heated water. It is used to incubate samples in water at a constant temperature over a long period of time

7-Centrifuge

A machine with a rapidly rotating container that applies

Centrifugal force to its contents.